BTEC Assignment Brief

| Qualification | Pearson BTEC International Level 3 Diploma in Engineering |
| --- | --- |
| Unit number and title | 19: Electronic Devices and Circuits |
| Learning aim(s) | B: Explore the safe operation and applications of digital logic devices and circuits that form the building blocks of commercial circuits. |
| Assignment title | Digital electronic devices and circuits |
| Assessor | Kelvin John |
| Issue date | 30 September 2022 |
| Hand in deadline | 15 December 2022 |
| Vocational Scenario or Context | You are working as a Junior Technician in a local company that specialises in repairing domestic electronic equipment. As part of your new role the owner has been providing you with opportunities to develop your skills and knowledge of electronic devices and circuits. They would now like you demonstrate that you can use a range of test equipment and software to investigate commonly used digital electronic devices and the type of circuits in which they are used. They have asked you keep accurate records of everything you do so that you can refer to them later as you build up your skills and knowledge base. |
| Task 1 | You have been asked to carry out a range of theoretical and practical investigations and evaluations into digital devices and circuits. You need to produce a report that includes information about the operation of a range of circuits, provide evidence that you have consistently worked safely in accordance with relevant risk assessments including comparisons between theoretical values, simulated values, and those measured practically. You will investigate the following: a) a combinational logic circuit; b) Two sequential logic circuits. a) Combinational logic circuit In a simple copy machine shown in Figure 1, a stop signal, S, is to be generated to stop the machine operation and energize an indicator light whenever either of the following conditions exists: (1) there is no paper in the paper feeder tray; or (2) the two microswitches in the paper path are activated, indicating a jam in the paper path. The presence of paper in the feeder tray is indicated by a HIGH at logic signal P. Each of the microswitches produces a logic signal (Q and R) that goes HIGH whenever paper is passing over the switch to activate it. Design the logic circuit to produce a HIGH at output signal S for the stated conditions. Figure 1 Use this information to write a Boolean expression for the truth table using AND, OR and NOT gates. You then need to: i. Construct a schematic diagram and simulate the operation of the logic circuit using standard logic gates. ii. Minimise the logic circuit using a Karnaugh Map, and then construct a schematic diagram, and simulate the operation of the minimised logic circuit, using standard logic gates. iii. Use De Morgan’s Theorems to convert the minimised circuit to use only NAND gates. iv. Build a prototype of your final circuit and verify that it meets the conditions in the truth table, and then compare the results from theory, simulation and measurement. b1) Sequential Logic Circuit – 3-bit asynchronous up/down counter Figure 2: 3 bit asynchronous up/down counter Table 1: 3 bit up/down counter chart You should: •From Figure 2, identify the pin out of a IC7476 dual JK flip-flop with negative edge trigger (or specify an equivalent that you have available) •Construct the schematic circuit of a 3 bit asynchronous Up/Down counter using flip flops using NI multisim. The IC7476 in the schematic has Set (PRE) and Reset (CLR) pins held low (logic 0) to allow changes to clock through to the output Q. (or give equivalent detail for JK available.) i. Use the schematic to simulate the behaviour of the circuit by using CLK as a manual clock. ii. Build a prototype circuit and verify that it works as expected, and then compare the results from theory, simulation and measurements. iii. What practical improvements would you make to the circuit? iv. Draw a schematic diagram to show how logic gates can be added to the Up/Down counter in the diagram to convert it into an up counter. v. Simulate the circuit designed in to demonstrate how the circuit can be used as an up or down counter. b2) Sequential Logic Circuit – 3-bit synchronous up counter Figure 3: 3 bit synchronous up counter Table 2: 3 bit up counter chart • Using Figure 3, identify the pin out of a IC7476 dual JK flip-flop with negative edge trigger (or specify an equivalent that you have available) • Construct the schematic circuit of a 3-bit Synchronous Counter using Flip-Flops using NI multisim. The IC7476 in the schematic has Set (PRE) and Reset (CLR) pins held low (logic 0) to allow changes to clock through to the output Q. i. Use the schematic to simulate the behaviour of the circuit by setting data values at the input and clocking them through. ii. Build a prototype of the circuit and demonstrate that it works as expected, and then compare the results from theory, simulation and measurements iii. What practical improvements would you make to the circuit? iv. Draw a schematic diagram to show how logic gates can be added to the Up counter in the diagram to convert it into an up/down counter. v. Simulate the circuit designed in to demonstrate how the circuit can be used as an up or down counter. |
| Checklist of evidence required for Task | A report organised into two separate sections, for the combinational logic circuit, and the two sequential circuits. The report should contain circuit diagrams, photographs, tables of results, sketches, screen shots, calculations and an evaluation of the physical and simulated circuits, supported by observation records and/or witness statements |
| Criteria covered by this task: |  |
| Unit/Criteria reference | To achieve the criteria, you must show that you are able to: |
| 19/ B.D2 | Evaluate the operation of at least one combinational logic circuit minimising the gates and two sequential logic circuits, comparing the results from safely and accurately conducted simulations and test. |
| 19/ B.M4 | Simulate, using accurately captured schematics, the correct operation of at least one combinational logic circuit minimising the gates and at least two sequential logic circuits. |
| 19/ B.M5 | Build at least one combinational logic circuit minimising the gates and at least two sequential logic circuits and test the characteristics of each one accurately. |
| 19/ B.M6 | Analyse, using the simulation and test results, the operation of at least three logic circuits. |
| 19/ B.P4 | Simulate, using captured schematics, the correct operation of at least one combinational logic circuit and two sequential logic circuits. |
| 19/ B.P5 | Build at least one combinational logic circuit and two sequential logic circuits safely and test the characteristics of each one. |
| 19/ B.P6 | Explain, using the simulation and test results, the operation of at least three logic circuits. |
| Sources of information to support you with this Assignment | Textbook Electronic devices; Floyd T L; Pearson (2007); ISBN-13: 978-0136155812 Digital fundamentals; Floyd T L; Pearson (2015); ISBN-13: 978-1292075983 Electronics; Sparkes JJ et al; Nelson Thornes Ltd (2003); ISBN 0 7487 7036 4 Websites electronics-tutorials.ws allaboutcircuits.com Further useful resources may be found at qualifications.pearson.com/en/support/published-resources.html#step1 |
| Other assessment materials attached to this Assignment Brief | Information, including circuit schematics, for two types of circuit that you need to investigate: a) One combinational logic circuit; and b) Two sequential logic circuits |