BTEC assignment brief

| Qualification | Pearson BTEC Level 5 Higher National Diploma in Engineering |
| --- | --- |
| Unit number and title | 5009: Further Programmable Logic Controllers (PLCs) |
| Learning Outcomes | LO1 Discuss the selection of a specific PLC for a given industrial application LO2 Evaluate how PLCs exchange information and process signals with other devices LO3 Design a PLC programme to solve an industrial process problem for a given application LO4 Analyse alternative strategies for using other types of programmable control devices in industrial applications. |
| Assignment title | Implementation of Smart Environmental Monitoring System |
| Assessor | Dr Neethu & Mr. Akbar |
| Hand-out date | 10/10/2026 |
| Hand-in deadline | 15/12/2026 |
| Vocational scenario or context | MedPack Solutions Ltd. is launching a high-throughput automated pharmaceutical packaging line for blister packs. As a Controls Engineer, you are responsible for selecting, implementing, and evaluating PLC-based automation. You will ensure robust signal processing, real-time control, and compliance with safety standards in a regulated manufacturing environment. |
| Task 1 | Task 1: PLC Selection and Justification - Technical Report You are required to submit a technical report recommending and justifying a suitable PLC model for the pill counting and packaging station. Use diagrams, IO tables, and datasheet extracts to support your analysis. Your report should address: - Process Description: Explain the full automation cycle: operator starts the count, pill sensor triggers relay, actuator packages pills, buzzer sounds for errors or completion, emergency stop and reset logic. Highlight any safety or reliability requirements, environmental factors, and regulatory basics. - Functional Requirements: Specify the number and type of digital inputs/outputs, relay output needs, voltage/current compatibility, panel mounting, and expansion potential. - Comparison of PLC Models: Analyse and compare at least three PLC models/manufacturers, focusing on digital IO capacity, relay outputs, expandability, programming features, user interface, reliability, and cost. Discuss practical factors: wiring, ease of installation, documentation, vendor support, and training resources. - Selection Justification: Defend your chosen PLC (with datasheet evidence) based on technical suitability, reliability, ease of programming, and future maintenance. Include a summary IO table and a simple block diagram of your proposed system layout. - Integration and Expansion: Discuss how the PLC will be incorporated into the station (including panel layout and wiring) and suggest how it could be expanded for future needs. - Critical Reflection: Briefly reflect on trade-offs (e.g., cost with specific cited numbers from vendors vs. features, support vs. flexibility) and any industry standards or safety considerations relevant to your selection. |
| Checklist of evidence required | - Process description with automation cycle explanation - Functional requirements specification (digital IO, relay outputs, voltage/current compatibility) - Comparison of at least three PLC models with feature analysis - Selection justification with datasheet evidence - Summary IO table and system block diagram - Integration and expansion discussion - Critical reflection on trade-offs and standards |
| Criteria covered by this task: |  |
| Unit/criteria reference | To achieve the criteria, you must show that you are able to: |
| LO1.P1 | Investigate the key industrial application characteristics of a given industrial application. |
| LO1.P2 | Compare the operational features and characteristics of PLCs from several manufacturers. |
| LO1.M1 | Justify the choice of a specific PLC suitable for a given industrial application. |
| LO1.D1 | Evaluate and justify the selection of a specific PLC for an industrial application. |

| Task 2 | Task 2: PLC Signal Processing and Communication - Project Demonstration Set up and demonstrate (live and recorded) the PLC's handling of digital signals from inputs and ouputs in the pill packaging station. Your demonstration should include: - Wiring and Configuration: Show correct wiring of switched inputs (start, reset, emergency stop, pill sensor) and relay outputs (actuator, buzzer). Explain how each input is debounced (if relevant), filtered, and mapped to the PLC program. - Operation Cycle: Run a typical packaging cycle: simulate operator actions, pill count detection, actuator activation, buzzer for errors or completion. Demonstrate PLC logic scanning, signal processing, and output actuation. - Error Simulation: Simulate common faults (e.g., sensor stuck, emergency stop pressed) and demonstrate system response, including buzzer activation and process halt. - Troubleshooting: Show basic fault diagnosis: how to test for wiring errors, input misreads, relay faults, and program logic errors. Discuss how signal integrity and noise could affect the system and mitigation strategies (shielded cables, input filters). - System Documentation: Briefly show your wiring diagram and IO allocation table to clarify system layout and logic, document findings in a lab record. |
| --- | --- |
| Checklist of evidence required | - Wiring diagrams with input/output connections - IO allocation table - Live demonstration of operation cycle (recorded) - Error simulation results - Troubleshooting procedures and findings - Lab record documentation - Discussion of signal integrity and mitigation strategies |
| Criteria covered by this task: |  |
| Unit/criteria reference | To achieve the criteria, you must show that you are able to: |
| LO2.P3 | Illustrate the main differences between communication links and standards used within PLC systems. |
| LO2.P4 | Review the advantages of using networked bus PLC systems. |
| LO2.M2 | Show how PLCs in industry integrate with HMIs and SCADA. |
| LO2.M3 | Evaluate the use of SCADA and HMIs in industry. |
| LO2.D2 | Provide a justified and valid rationale for the convergence of PLCs/HMIs and SCADA control systems. |

| Task 3 | Task 3: PLC Programming Implementation – Offline Programming Test Develop programs for PLC control subtasks in the medicine packaging plant. Your programming submission should include: - Program Structure: Implement start/stop control, pill count verification, actuator sequencing, buzzer alert logic (e.g., completion, error, emergency stop). Use basic logic functions (AND, OR, NOT), timers/counters for pill counting, and program comments for clarity. - Advanced Features: If supported, demonstrate use of software timers, counters, or simple memory bits for process state tracking. Show program test and debug features, such as monitoring live values and forcing IO for diagnostics. - Live Testing: Run the program through typical cycles, simulate input changes, and handle errors in real time and document your findings. Make at least one live logic modification or fault correction during the test. - Rationale and Reflection: Explain your programming approach, logic choices, and safeguards for safe operation. Reflect briefly on challenges (e.g., debouncing, timing, state management), how you resolved them, and lessons learned for future programming tasks. |
| --- | --- |
| Checklist of evidence required | - PLC program code with comments and documentation - Program logic diagrams - Live testing demonstration (recorded) - Test results and findings - Logic modifications and fault corrections - Rationale and reflection on programming approach - Challenges and solutions documentation |
| Criteria covered by this task: |  |
| Unit/criteria reference | To achieve the criteria, you must show that you are able to: |
| LO3.P5 | Design a PLC programme to solve an industrial application problem. |
| LO3.P6 | Demonstrate the use of PLC programming and simulation software in a given application. |
| LO3.M4 | Demonstrate the use of test and debug software to correct PLC program faults. |
| LO3.M5 | Explore the practical uses of PLC advanced functions. |
| LO3.D3 | Critically evaluate a PLC programme used to solve an industrial application problem. |

| Task 4 | Task 4: Alternative Automation Strategies - Professional Discussion Participate in a professional oral discussion (live and recorded) critically evaluating alternatives to PLCs for similar automation tasks. Your discussion should include: - Alternatives Overview: Defend at least two alternative control devices (e.g., microcontroller such as Nordic's nRF52, traditional relay logic panel). - Comparison: Discuss wiring complexity, reliability, programming difficulty, scalability, cost, and suitability for simple digital IO tasks. Evaluate regulatory, safety, and maintenance considerations for each alternative. Assess pros and cons for small-batch pharmaceutical automation and potential for future expansion. - Scenario Application: Given a scenario (e.g., pill count and packaging, with added safety interlocks), justify which control approach is optimal and why. Discuss circumstances where a PLC might not be the best choice and how alternatives could be adapted. - Critical Reflection: Reflect on broader industry trends: the role of simple PLCs, microcontrollers, and relay logic in modern manufacturing. Consider future skills and learning needed to adapt to changing automation technologies. |
| --- | --- |
| Checklist of evidence required | - Overview of at least two alternative control devices - Comparative analysis of PLC vs. alternatives (wiring, reliability, programming, scalability, cost) - Regulatory, safety, and maintenance evaluation - Scenario application analysis and justification - Critical reflection on industry trends and future skills - Live and recorded discussion evidence |
| Criteria covered by this task: |  |
| Unit/criteria reference | To achieve the criteria, you must show that you are able to: |
| LO4.P7 | Review the different types of programmable control devices available. |
| LO4.P8 | Examine an industrial application to determine the required characteristics of a control device. |
| LO4.M6 | Review the problems faced by using alternative devices in an industrial environment. |
| LO4.D4 | Critically evaluate the selection of an alternative programmable device in a given application. |

| Sources of information to support you with this assignment | Bolton, W. (2015) Programmable Logic Controllers. 5th Ed. Newes. Kamel, K. and Kamel, E. (2013) Programmable Logic Controllers: Industrial Control. McGraw-Hill Education. Morton, J. (2005) The PIC Microcontroller: Your Personal Introductory Course. 3rd Ed. Newnes. Perez, E. (2012) Introduction to PLCs: A beginner's guide to Programmable Logic Controllers. Elvin Perez Adrover. Petruzella F. (2023) Programmable Logic Controllers. 6th Ed. McGraw Hill. Rehg A. R. and SARTOR J. G. (2014) Programmable Logic Controllers. 2nd Ed. Pearson. Stewart G. R. (2021) Plc Programming for Beginners. SIEMENS. |
| --- | --- |

| Submission Format | Report: Document to be submitted in .ODT open document format, with font preferably as open sans in size 12. Code: Project file folders in .ZIP format from OpenPLC Editor. Professional Discussion Notes: Documentation of the discussion with the assessor regarding simulation, implementation. Presentation Slides: Presentation as .ODP open document format or .PDF. All references must follow the Harvard referencing style |
| --- | --- |