# Manufacturing Lab Process-Based Risk Assessment Register

This document provides a process-based Manufacturing Lab risk assessment register using the format: SL.NO, Process / Practical Title, ID, Activity/Hazard Source, Potential Risks / Hazards, Risk Level, Control Measures / Mitigation, and Residual Risk.

## 1. PCB Artwork Preparation and Transfer Setup

| SL.NO | Process / Practical Title | ID | Activity/Hazard Source | Potential Risks / Hazards | Risk Level | Control Measures / Mitigation | Residual Risk |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | PCB Artwork Preparation and Transfer Setup | 1 | Wrong board dimensions, mirrored print, or incorrect layer orientation | Scrapped board, repeated rework, rushed unsafe correction steps later in the process | Low | Check dimensions, orientation, and print scaling before exposing or transferring artwork | Low |
| 1 | PCB Artwork Preparation and Transfer Setup | 2 | Cutting, trimming, or handling copper-clad board edges | Minor cuts, splinters, sharp-edge handling injury | Medium | Handle board edges carefully; use stable support when trimming; store blanks flat and safely | Low |
| 2 | Cleaning PCB Surface Before Exposure or Assembly | 3 | Solvent or abrasive cleaning of copper surface | Skin irritation, eye irritation, spread of contaminated wipes, fire risk from solvent misuse | Medium | Use labeled cleaning materials only; ventilate the area; keep wipes away from ignition sources; wash hands after use | Low |

## 2. UV Exposure and Photoresist PCB Processing

| SL.NO | Process / Practical Title | ID | Activity/Hazard Source | Potential Risks / Hazards | Risk Level | Control Measures / Mitigation | Residual Risk |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | UV Exposure of Photoresist PCB | 1 | UV box used with cover open, damaged, or bypassed | Eye or skin exposure to UV radiation | High | Keep UV cover closed and intact; do not defeat safety features; remove faulty unit from service | Low |
| 1 | UV Exposure of Photoresist PCB | 2 | Awkward setup while arranging board and artwork near energised unit | Unsafe hand movement, poor control, repeated exposure setup errors | Medium | Prepare board, artwork, and frame before energising the UV unit; avoid rushed adjustments during exposure | Low |
| 2 | Developing or Handling Sensitised PCB Material | 3 | Mishandling prepared board surfaces or chemical residues | Contamination, skin irritation, failed board leading to rework | Medium | Handle prepared boards deliberately; avoid touching active surfaces unnecessarily; follow chemical handling controls where applicable | Low |

## 3. Ferric Chloride Etching, Rinsing, and Waste Handling

| SL.NO | Process / Practical Title | ID | Activity/Hazard Source | Potential Risks / Hazards | Risk Level | Control Measures / Mitigation | Residual Risk |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Ferric Chloride Etching | 1 | Contact with etchant during immersion, transfer, or agitation | Chemical splash to skin or eyes, staining, corrosive contact | High | Confirm chemical identity; wear splash eye protection and compatible gloves; handle trays and containers carefully | Low |
| 1 | Ferric Chloride Etching | 2 | Crowded or restricted wet-chemical area | Collision, spill, blocked egress, poor emergency response | High | Keep immediate area to one active operator where space is limited; keep access and exit route clear | Low |
| 1 | Ferric Chloride Etching | 3 | Splash or spill during agitation or board removal | Contamination of benches, floor, tools, and clothing | High | Lower and remove boards carefully; avoid jerking trays; contain and clean spills by approved method immediately | Low |
| 2 | PCB Rinsing and Post-Etch Cleanup | 4 | Contaminated rinse water and stained tools carried into clean areas | Spread of contamination, unsafe disposal, secondary skin contact | High | Use designated rinse and waste route only; clean touch points and tools after use; remove gloves safely | Low |
| 3 | Chemical Waste Handling After Etching | 5 | Incorrect disposal of etchant, wipes, gloves, or rinse waste | Environmental contamination, unsafe waste exposure, non-compliant disposal | High | Segregate and dispose of all process waste by the approved chemical waste route only | Low |

## 4. PCB Drilling, Cutting, and Mechanical Finishing

| SL.NO | Process / Practical Title | ID | Activity/Hazard Source | Potential Risks / Hazards | Risk Level | Control Measures / Mitigation | Residual Risk |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | PCB Drilling | 1 | Hands too close to rotating drill bit or chuck | Puncture injury, laceration, trapped fingers | High | Keep hands clear of spindle zone; use backing board, clamp, or stable support; never reach under moving bit | Low |
| 1 | PCB Drilling | 2 | Brittle drill bit breakage | Flying fragments, eye injury | High | Wear eye protection; use correct feed pressure; inspect and replace damaged bits promptly | Low |
| 1 | PCB Drilling | 3 | Cleaning swarf or repositioning board before full stop | Contact injury, snagging, loss of control | Medium | Wait for complete spindle stop before cleaning, repositioning, or measurement | Low |
| 2 | PCB Cutting, Edge Trimming, or Board Separation | 4 | Sharp board edges, cutters, or improvised trimming method | Cuts, punctures, broken board fragments | Medium | Use the correct cutting method and support the board properly; handle trimmed edges carefully | Low |
| 3 | Deburring and Dust Cleanup | 5 | Fine PCB dust, sharp fragments, and debris cleanup | Eye irritation, respiratory irritation, contamination of bench area | Medium | Use controlled cleanup method; do not blow dust toward people; clean bench fully after drilling or finishing | Low |

## 5. Through-Hole PCB Assembly and Soldering

| SL.NO | Process / Practical Title | ID | Activity/Hazard Source | Potential Risks / Hazards | Risk Level | Control Measures / Mitigation | Residual Risk |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Component Placement and PCB Assembly | 1 | Incorrect component orientation, polarity, or placement | Rework demand, damaged components on power-up, rushed correction work | Medium | Verify component values, orientation, and polarity before soldering permanent joints | Low |
| 2 | Through-Hole Soldering | 2 | Contact with hot iron, hot joint, or recently heated lead | Burns to hands or skin | High | Use insulated handle only; return iron to stand; allow joint and board to cool before handling | Low |
| 2 | Through-Hole Soldering | 3 | Molten solder splash or clipped lead ejection | Eye injury, minor burns, puncture hazard | Medium | Wear eye protection; control cut direction; collect clipped leads immediately | Low |
| 2 | Through-Hole Soldering | 4 | Excessive heat applied to pads, tracks, or component leads | Lifted pads, scorched PCB, damaged components, increased burn risk | Medium | Use the lowest practical heat setting and sound soldering technique; do not dwell excessively on one joint | Low |
| 3 | Flux Residue and Lead Contamination Control | 5 | Handling flux residues, solder, and contaminated surfaces | Hand contamination, ingestion risk, spread to notebooks or personal items | Medium | Wash hands after work; keep food and drink out of the bench area; clean contaminated surfaces after assembly | Low |

## 6. Desoldering, Rework, and Hot-Air Repair

| SL.NO | Process / Practical Title | ID | Activity/Hazard Source | Potential Risks / Hazards | Risk Level | Control Measures / Mitigation | Residual Risk |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Desoldering and Component Removal | 1 | Heated joint release, braid use, or pump discharge | Burns, solder splash, fume exposure | Medium | Use eye protection; keep hands clear of hot joint area; ventilate and dispose of removed solder safely | Low |
| 2 | Hot-Air Rework of PCB Assemblies | 2 | Contact with hot nozzle or high-temperature airflow | Burns, heat injury | High | Keep nozzle away from skin; return tool to holder; treat nozzle as hot until fully cooled | Low |
| 2 | Hot-Air Rework of PCB Assemblies | 3 | Excessive heat or airflow on local area | PCB scorching, blown components, damaged connectors or plastics | High | Start with conservative settings; shield nearby components where needed; keep nozzle moving | Low |
| 2 | Hot-Air Rework of PCB Assemblies | 4 | Fumes from flux, conformal residue, plastics, or overheating board | Respiratory irritation, unpleasant exposure | Medium | Use ventilation or fume extraction; stop if unusual smoke or strong odour develops | Low |
| 3 | Rework Inspection and Restart | 5 | Restarting or retesting board before confirming rework quality | Repeat failure, further overheating, more difficult fault recovery | Medium | Inspect solder bridges, lifted pads, and nearby components before any further process step or energisation | Low |

## 7. PCB Inspection, Continuity Checks, and Bench Verification

| SL.NO | Process / Practical Title | ID | Activity/Hazard Source | Potential Risks / Hazards | Risk Level | Control Measures / Mitigation | Residual Risk |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Visual Inspection of Finished PCB | 1 | Close handling of sharp leads, rough board edges, or partially cooled joints | Minor cuts, puncture injury, burn from recently soldered joints | Medium | Confirm board has cooled; handle by safe edges; trim and dispose of sharp waste before inspection | Low |
| 2 | Continuity or Resistance Checks on Fabricated PCB | 2 | Testing before stored charge or connected supply is removed | Meter damage, false readings, accidental short circuit | Medium | Perform checks on isolated, unpowered boards only; discharge any stored energy before testing | Low |
| 3 | Functional Bench Verification of New PCB Assembly | 3 | First energisation of incorrectly assembled board | Hot components, smoke event, damaged PCB, repeated fault chasing | High | Use current-limited first power-up where applicable; inspect polarity and soldering quality before energisation; stop immediately on abnormal heat or smell | Low |

## 8. 3D Print File Preparation and Printer Setup

| SL.NO | Process / Practical Title | ID | Activity/Hazard Source | Potential Risks / Hazards | Risk Level | Control Measures / Mitigation | Residual Risk |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Slicing and Print File Preparation | 1 | Wrong material profile, temperature, or support settings | Failed print, nozzle clog, overheated filament, excessive fumes | Medium | Confirm filament type and slicing settings before start; do not use unknown or unlabeled filament | Low |
| 2 | Printer Setup and Filament Loading | 2 | Loading or unloading filament near heated hotend | Burn injury, snagged filament, forced jam clearing | Medium | Follow the correct loading sequence; keep hands clear of heated areas; do not force filament through resistance | Low |
| 3 | Build Plate Preparation and Print Start | 3 | Loose debris, tools left inside enclosure, or poor bed preparation | Poor adhesion, nozzle collision, sudden print failure | Medium | Clear the enclosure fully; prepare bed correctly; observe the first layer and stop early if adhesion fails | Low |

## 9. Closed-Body 3D Printing Operation and Print Removal

| SL.NO | Process / Practical Title | ID | Activity/Hazard Source | Potential Risks / Hazards | Risk Level | Control Measures / Mitigation | Residual Risk |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Closed-Body 3D Printing Operation | 1 | Contact with hot nozzle, hotend, heated bed, or hot printed part | Burns to hands or skin | High | Do not touch hotend or bed during or just after printing; allow cooling before intervention or part removal | Low |
| 1 | Closed-Body 3D Printing Operation | 2 | Reaching into enclosure during live print | Pinch, strike, trapped fingers, contact with moving axes | Medium | Keep hands clear of motion envelope; do not reach in during live print; stop or pause the printer first | Low |
| 1 | Closed-Body 3D Printing Operation | 3 | Long unattended heated operation or electrical fault | Fire risk, overheated material, equipment damage | High | Keep combustibles clear; use agreed monitoring arrangements; know emergency isolation method before starting long prints | Low |
| 2 | Print Failure Response and Part Removal | 4 | Tangled print, nozzle buildup, or forced part removal | Hot plastic contact, tool slip, damaged bed surface, sudden part release | Medium | Stop the print and allow a safe state before clearing faults; remove parts with controlled force and correct tools only | Low |
| 3 | Fume and Material Control During Printing | 5 | Heated filament emissions, unknown polymer, or wrong temperature | Respiratory irritation, discomfort, degraded print material | Medium | Operate in a well-ventilated area; verify material and temperature settings; stop if unusual fumes or smell occur | Low |

## 10. Post-Processing, Finishing, and Prototype Integration

| SL.NO | Process / Practical Title | ID | Activity/Hazard Source | Potential Risks / Hazards | Risk Level | Control Measures / Mitigation | Residual Risk |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Removal of Supports, Strings, and Printed Waste | 1 | Sharp tools, snapped plastic, or brittle support removal | Cuts, puncture injuries, flying fragments | Medium | Cut away supports carefully; wear eye protection where fragments may eject; hold part securely | Low |
| 2 | Fitting Printed Parts to PCB or Prototype Assemblies | 2 | Misalignment, forcing parts together, or trapping wires during assembly | Broken parts, damaged PCB, pinched fingers, insulation damage | Medium | Dry-fit parts first; route wires deliberately; do not force assemblies that do not align cleanly | Low |
| 3 | Final Bench Cleanup and Waste Removal | 3 | Leaving solder waste, dust, failed prints, clipped leads, or chemical residues behind | Secondary injury to next user, contamination spread, fire or housekeeping risk | Medium | Treat cleanup as part of the task; collect sharp waste and print waste; leave the bench and process area safe for the next user | Low |

## 11. Additional Similar Processes to Add If Present

These are not confirmed as present. They are suggested for inclusion if used in the Manufacturing Lab.

| SL.NO | Process / Practical Title | ID | Activity/Hazard Source | Potential Risks / Hazards | Risk Level | Control Measures / Mitigation | Residual Risk |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A1 | Surface-Mount PCB Assembly | 1 | Small component handling, tweezers, solder paste, and close hot-work | Burns, eye strain, solder bridging, dropped components | Medium | Use stable workholding, controlled heat, and good lighting; inspect joints before rework | Low |
| A2 | PCB Track Repair or Jumper Rework | 2 | Scraping solder mask, fine soldering, or exposed copper repair | Cuts, lifted tracks, accidental short circuits | Medium | Work slowly with magnification where needed; confirm isolation and continuity after repair | Low |
| A3 | 3D Printed Enclosure or Fixture Production | 3 | Large or long-duration print jobs for enclosures and jigs | Long heat exposure, warping, failed print accumulation, fire risk if poorly monitored | Medium | Verify file and material settings carefully; confirm supervision arrangements before long prints | Low |
| A4 | Solvent Cleaning or Adhesive Bonding of Printed Parts | 4 | Use of IPA, cleaners, or adhesives during finishing | Skin irritation, fumes, eye contact, fire risk | Medium | Use labeled products only; ventilate area; keep away from ignition sources; wear suitable PPE | Low |

## Final Requirement

This register is a Manufacturing Lab process-based risk assessment summary and should be used alongside the Manufacturing Lab equipment risk assessment register, SOPs, supervision arrangements, and local institutional controls.