# Manufacturing Lab Risk Assessment Register

This document provides a Manufacturing Lab-specific risk assessment register using the format: SL.NO, Equipment name, ID, Activity/Hazard Source, Potential Risks / Hazards, Risk Level, Control Measures / Mitigation, and Residual Risk.

## 1. Soldering Iron and 80/20 Solder

| SL.NO | Equipment name | ID | Activity/Hazard Source | Potential Risks / Hazards | Risk Level | Control Measures / Mitigation | Residual Risk |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Soldering Iron and 80/20 Solder | 1 | Contact with hot tip, barrel, or recently soldered joint | Burns to hands or skin | High | Use insulated handle only; return iron to stand; allow cooling time before handling workpiece | Low |
| 1 | Soldering Iron and 80/20 Solder | 2 | Molten solder splash | Eye injury, minor burns, damage to clothing | Medium | Wear eye protection; keep face away from joint; control movement of work and tip | Low |
| 1 | Soldering Iron and 80/20 Solder | 3 | Overheating with manual power knob | Excess fumes, board damage, lifted pads, increased burn risk | Medium | Use lowest practical heat setting; do not substitute excess temperature for poor technique | Low |
| 1 | Soldering Iron and 80/20 Solder | 4 | Flux fumes and heated residue | Eye and respiratory irritation | Medium | Use fume extraction or ventilation; avoid breathing zone above joint | Low |
| 1 | Soldering Iron and 80/20 Solder | 5 | Lead-containing solder or contaminated residues | Hand contamination, ingestion risk | Medium | Wash hands after work; no food or drink at bench; control contaminated waste and residues | Low |
| 1 | Soldering Iron and 80/20 Solder | 6 | Iron left out of stand or near combustibles | Fire risk, accidental burns | High | Always return to stable stand; keep flammable materials out of hot-work zone | Low |
| 1 | Soldering Iron and 80/20 Solder | 7 | Trimming sharp component leads | Eye injury, puncture hazard | Medium | Wear eye protection; collect trimmings immediately; control direction of clipped leads | Low |

## 2. Hot Air Rework Gun

| SL.NO | Equipment name | ID | Activity/Hazard Source | Potential Risks / Hazards | Risk Level | Control Measures / Mitigation | Residual Risk |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | Hot Air Rework Gun | 1 | Contact with hot nozzle or airflow | Burns, heat injury | High | Keep nozzle away from skin; return tool to holder; treat nozzle as hot until fully cooled | Low |
| 2 | Hot Air Rework Gun | 2 | Excessive temperature or airflow | PCB scorching, blown components, damaged plastics | High | Start with conservative settings; increase only as required; shield nearby sensitive parts | Low |
| 2 | Hot Air Rework Gun | 3 | Molten solder movement under airflow | Solder splash, displaced small components | Medium | Wear eye protection; avoid excessive airflow; control nozzle distance and movement | Low |
| 2 | Hot Air Rework Gun | 4 | Wider-area heating than intended | Damage to adjacent components, connectors, labels, wire insulation | Medium | Plan surrounding clearance; use shielding where needed; keep nozzle moving | Low |
| 2 | Hot Air Rework Gun | 5 | Fume generation from flux or plastics | Respiratory irritation, unpleasant exposure | Medium | Use ventilation or fume extraction; stop if unusual smoke or strong odour develops | Low |
| 2 | Hot Air Rework Gun | 6 | Skipping cooldown stage | Damaged tool, uncontrolled hot nozzle left in area | Medium | Follow proper cooldown process; do not store or unplug prematurely if cooldown is required | Low |

## 3. PCB Drill Press

| SL.NO | Equipment name | ID | Activity/Hazard Source | Potential Risks / Hazards | Risk Level | Control Measures / Mitigation | Residual Risk |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | PCB Drill Press | 1 | Contact with rotating drill bit or chuck | Puncture injury, laceration | High | Keep hands clear of spindle zone; maintain safe hand position; never reach beneath moving bit | Low |
| 3 | PCB Drill Press | 2 | Holding PCB too close by hand | Board spinning or pulling fingers into danger | High | Use backing board and clamp or stable support; keep fingers well clear of bit path | Low |
| 3 | PCB Drill Press | 3 | Small brittle bit breakage | Flying fragments, eye injury | High | Eye protection mandatory; use correct feed pressure; inspect and replace damaged bits | Low |
| 3 | PCB Drill Press | 4 | PCB dust and small fragments | Eye irritation, respiratory irritation, contamination | Medium | Control dust during cleanup; do not blow dust toward people; clean bench after use | Low |
| 3 | PCB Drill Press | 5 | Loose clothing or hair near spindle | Entanglement with rotating tool | High | Tie back hair; remove loose items; keep sleeves and lanyards away from moving parts | Low |
| 3 | PCB Drill Press | 6 | Clearing swarf before full spindle stop | Contact injury, snagging on tool | Medium | Stop machine fully before cleaning or repositioning work | Low |

## 4. PCB UV Exposure and Ferric Chloride Etching Process

| SL.NO | Equipment name | ID | Activity/Hazard Source | Potential Risks / Hazards | Risk Level | Control Measures / Mitigation | Residual Risk |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | PCB UV Exposure and Ferric Chloride Etching Process | 1 | UV exposure box operated with cover open or faulty | Eye or skin exposure to UV radiation | High | Keep cover closed and intact; do not defeat safety features; remove faulty UV unit from service | Low |
| 4 | PCB UV Exposure and Ferric Chloride Etching Process | 2 | Ferric chloride or similar etchant contact | Chemical splash to skin or eyes, staining, corrosive contact | High | Confirm chemical identity; use splash eye protection and compatible gloves; handle containers carefully | Low |
| 4 | PCB UV Exposure and Ferric Chloride Etching Process | 3 | Transfer and agitation of etchant | Spill or splash event, contamination of work area | High | Lower boards carefully; avoid jerking containers; keep stable trays and lids; clean spills immediately by approved method | Low |
| 4 | PCB UV Exposure and Ferric Chloride Etching Process | 4 | Restricted wet-chemical corner occupancy | Collisions, blocked egress, poor spill response | High | One active operator at a time in immediate area; keep access route clear; sequence work instead of crowding | Low |
| 4 | PCB UV Exposure and Ferric Chloride Etching Process | 5 | Chemical waste and contaminated rinse water | Unsafe disposal, environmental contamination, spread to clean benches | High | Use approved waste route only; segregate contaminated wipes, gloves, and rinse waste; clean touch points afterward | Low |
| 4 | PCB UV Exposure and Ferric Chloride Etching Process | 6 | Poor ventilation during chemical work | Irritation from vapours or strong odours, discomfort in restricted area | Medium | Use suitable ventilation; stop if area becomes congested, poorly ventilated, or difficult to leave safely | Low |

## 5. Closed-Body 3D Printer

| SL.NO | Equipment name | ID | Activity/Hazard Source | Potential Risks / Hazards | Risk Level | Control Measures / Mitigation | Residual Risk |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | Closed-Body 3D Printer | 1 | Contact with hot nozzle, hotend, or heated bed | Burns to hands or skin | High | Do not touch hotend or bed during or just after printing; allow cooling before intervention | Low |
| 5 | Closed-Body 3D Printer | 2 | Moving axes and print head during operation | Pinch, strike, trapped fingers | Medium | Keep hands clear of motion envelope; do not reach in during live print; stop printer first | Low |
| 5 | Closed-Body 3D Printer | 3 | Failed print with tangled material or nozzle collision | Hot plastic buildup, tool contact while moving | Medium | Stop or pause machine before clearing; observe first layers and monitor at intervals | Low |
| 5 | Closed-Body 3D Printer | 4 | Heated filament fumes and ultrafine emissions | Respiratory irritation, discomfort | Medium | Operate in well-ventilated area; confirm material and suitable temperatures; stop if unusual fumes occur | Low |
| 5 | Closed-Body 3D Printer | 5 | Long unattended heated operation | Fire risk from electrical or thermal fault | High | Keep printer area clear of combustibles; use agreed monitoring arrangements; know emergency isolation method | Low |
| 5 | Closed-Body 3D Printer | 6 | Unknown filament or wrong print temperature | Nozzle clog, degraded material, excessive fumes, print failure | Medium | Use labeled filament only; verify material settings before print start | Low |

## Additional Equipment to Add If Present

These are not confirmed as present. They are suggested for inclusion if used in the Manufacturing Lab.

| SL.NO | Equipment name | ID | Activity/Hazard Source | Potential Risks / Hazards | Risk Level | Control Measures / Mitigation | Residual Risk |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A1 | Desolder Pump / Desolder Braid | 1 | Heated desoldering and rapid release of solder | Burns, solder splash, fume exposure | Medium | Use eye protection; control hot joint access; ventilate area; dispose of waste safely | Low |
| A2 | PCB Guillotine / Hand Shear | 1 | Cutting FR boards or thin material | Finger pinch, cuts, flying fragments | Medium | Keep fingers clear of cut path; wear eye protection; support work properly | Low |
| A3 | Flush Cutters / Side Cutters | 1 | Trimming leads and wire ends | Flying fragments, cuts, puncture injuries | Medium | Wear eye protection; control cut direction; store tools safely after use | Low |
| A4 | Adhesive / Solvent Cleaning Chemicals | 1 | Use of IPA, cleaners, or adhesives during fabrication | Skin irritation, fumes, fire risk, eye contact | Medium | Use labeled products only; ventilate area; keep away from ignition sources; wear appropriate PPE | Low |

## Final Requirement

This register is a Manufacturing Lab risk assessment summary and should be used alongside the Manufacturing Lab SOPs, supervision arrangements, and local institutional controls.