BTEC assignment brief

| Qualification | Pearson BTEC Level 4 Higher National Diploma in Engineering |
| --- | --- |
| Unit number and title | 4015: Automation, Robotics and Programmable Logic Controllers (PLCs) |
| Learning Outcomes | LO1 Describe the design and operational characteristics of a PLC system LO4 Investigate the design and safe operation of a robot within an industrial application |
| Assignment title | PLC Systems and Robotic Cell Safety in an Automated Manufacturing Line |
| Assessor | Ms Anu |
| Hand-out date | 10/03/2026 |
| Hand-in deadline | 15/15/2026 |
| Vocational scenario or context | Axis Integrated Manufacturing Ltd. is upgrading an automated production cell used for sorting, handling and packaging components. The cell combines a PLC-controlled conveyor and sensing system with an articulated industrial robot operating inside a guarded work area. The company requires a technical investigation into the PLC system used to control the line and the safety arrangements needed for the robotic cell. As a trainee automation engineer, you are required to describe the design and operational characteristics of the PLC system and investigate the design and safe operation of the robotic cell. Your work must show clear understanding of PLC types, input and output devices, communication links, PLC architecture and programming languages, together with robotic cell safety systems, operating envelope, risk assessment and safe working procedures. |
| Task 1 | PLC System Characteristics and Operational Review – Presentation You are required to deliver a presentation explaining the PLC system used in the automated manufacturing line. Your presentation should address: - Process overview: describe the automated line and the role of the PLC in controlling sensors, actuators, conveyor sections and interlocks. - PLC types and configurations: describe the key differences of PLC types and configurations, and their typical applications. Include modular, unitary and rack-mounted systems. - Input and output devices: determine the types of PLC input and output devices available for the system. Include digital and analogue devices with suitable industrial examples. - Communication links: describe the different types of communication links used with PLCs. Include RS-232, RS-422, RS-485, Ethernet and industrial network examples. - Programming languages: explain the different types of PLC programming languages available in accordance with IEC 61131-3. - Internal architecture: analyse the internal architecture of a typical PLC to determine its operational applications. Include CPU, memory, power supply, interface modules, scan cycle and input/output processing. You should support your presentation with slides, diagrams, tables and technical explanation. |
| Checklist of evidence required | - Presentation slides - Live presentation delivery - Description of the automated application and the role of the PLC - Description of the key differences of PLC types and configurations, and their typical applications - Determination of the PLC input and output devices available - Description of the different communication links used with PLCs - Explanation of different PLC programming languages - Analysis of the internal architecture of a typical PLC to determine operational applications |
| Criteria covered by this task: |  |
| Unit/criteria reference | To achieve the criteria, you must show that you are able to: |
| LO1.P1 | Describe the key differences of PLC types and configurations, and their typical applications. |
| LO1.P2 | Determine the types of PLC input and output devices available. |
| LO1.P3 | Describe the different types of communication links used with PLCs. |
| LO1.M1 | Explain the different types of PLC programming languages available. |
| LO1.D1 | Analyse the internal architecture of a typical PLC to determine its operational applications. |

| Task 2 | Task 2: Circuit Performance Improvement and Reliability-Safe Modification Proposal — Design Portfolio What you must produce A design portfolio section (report + figures) where you justify performance improvement measures and propose design modifications that improve functionality and maintenance without creating reliability issues. Your submission must address 1. Baseline performance and limitation(s) - Identify and evidence performance issues (examples: slow cycle time, pressure drops, overshoot, inefficiency, excessive air/oil loss, heat). - Provide baseline evidence: simulation results, measured data, or manufacturer-based estimates. 2. Performance improvement measures - Justify measures taken to improve circuit design in respect of performance. - Minimum expectation: 3 improvements, each with: - what is changed (component, setting, layout, control approach) - why it improves performance (link to pressure/flow/control principles) - evidence (before/after metric such as cycle time, pressure stability, energy/air use) 3. Design modifications for functionality + maintenance without reliability issues - Propose modifications that improve: - functionality (control stability, repeatability, capability) - maintenance (accessibility, isolation points, test points, modularity, documentation, monitoring) - You must also show why reliability is not reduced: - identify any new failure modes introduced - provide mitigations (e.g., rated fittings, redundancy, proof testing, contamination control) 4. Alternative actuating systems (LO3 descriptor coverage) - Briefly compare an alternative actuating approach relevant to the application (e.g., electric servo, electromechanical actuator, different fluid power topology) and explain when it would be preferable. |
| --- | --- |
| Checklist of evidence required | - Baseline vs improved performance evidence (before/after) - 3+ performance improvements with clear justification - Proposed modifications with maintainability benefit + reliability risk control - Alternative actuation comparison - Updated circuit/architecture diagrams (where relevant) - References (Harvard) |
| Criteria covered by this task: |  |
| Unit/criteria reference | To achieve the criteria, you must show that you are able to: |
| LO3.M3 | Justify the measures taken to improve circuit design in respect of performance. |
| LO3.D3 | Propose the design modifications that can be introduced to improve the functionality and maintenance of pneumatic and hydraulic systems without creating reliability issues. |

| Task 3 | Task 3: Maintenance Investigation and Fault-Finding Evaluation — Maintenance Report + Discussion Evidence What you must produce A maintenance-focused report (plus supporting logs/templates) that recognises faults and hazards, investigates efficient maintenance procedures, compares pneumatic vs hydraulic safety/legislation aspects, and evaluates how maintenance/testing/fault-finding improves performance. If your centre uses professional discussion, include brief notes/slides and an assessor observation record. Your submission must address 1. Faults and hazards - Recognise system faults and potential hazards for pneumatic and hydraulic systems: - leaks, pressure loss, seal wear, contamination, valve sticking, hose failure, compressor/pump issues, sensor faults - hazards: stored energy release, hose whip, injection injury (hydraulic), noise (pneumatic), slips/environmental contamination, pinch/crush points - Provide a fault table: symptom → cause → effect → immediate action. 2. Efficient maintenance procedures and records - Investigate procedures to ensure efficient maintenance and operation: - inspection schedules (daily/weekly/monthly) - lubrication/fluid management strategy - contamination control, filter management, isolation/LOTO steps - record-keeping: maintenance log sheets, checklists, fault reports - Include at least one completed example record (or a realistic sample). 3. Legislation and safety comparison - Compare construction and operation of hydraulic and pneumatic systems with regards to legislation and safety issues. - Minimum expectation: a comparison table linking: - safety risks (pressure, leaks, noise, cleanliness) - inspection/testing requirements - relevant standards/regulations used by your centre/region (e.g., pressure system safety requirements, ISO symbol standards) 4. Problem-solving tools - Use at least two tools on one selected fault: - Root Cause Analysis (RCA / 5 Whys) - Fishbone diagram - PFMEA (recommended for strongest LO4 alignment) - Optionally mention where PPS/APQP would fit in an industrial lifecycle. 5. Performance evaluation - Evaluate the importance of maintenance, inspection, testing and fault-finding in respect of improved system performance: - link actions to measurable outcomes (downtime, cycle time stability, scrap rate, safety incidents, component life) - explain why the improvements occur (contamination reduction, early detection, correct settings) 6. Job market and opportunities - Briefly describe roles and efficiency opportunities related to maintaining pneumatic/hydraulic systems (maintenance technician, reliability engineer, automation maintenance). |
| --- | --- |
| Checklist of evidence required | - Fault + hazard recognition table - Maintenance procedures + schedules + record templates - Pneumatic vs hydraulic legislation/safety comparison - RCA/Fishbone + (preferably) PFMEA evidence - Evaluation linking maintenance/testing/fault-finding to performance - Job market/opportunities short section - References (Harvard) |
| Criteria covered by this task: |  |
| Unit/criteria reference | To achieve the criteria, you must show that you are able to: |
| LO4.P8 | Recognise system faults and potential hazards in pneumatic and hydraulic systems. |
| LO4.P9 | Investigate procedures to ensure efficient maintenance and operation of pneumatic and hydraulic systems. |
| LO4.M4 | Compare construction and operation of hydraulic and pneumatic systems with regards to legislation and safety issues. |
| LO4.D4 | Evaluate the importance of maintenance, inspection, testing and fault-finding in respect of improved system performance. |

| Sources of information to support you with this assignment | Bolton, W. Pneumatic and Hydraulic Systems Merritt, H. Hydraulic Control Systems ISO 1219-1 / ISO 1219-2 (symbols/circuit diagrams) Pressure systems safety guidance applicable to your centre/region Manufacturer datasheets (cylinders, valves, FRL, pumps, filters) Simulation tools: FluidSIM / Simscape Fluids / MATLAB-Simulink |
| --- | --- |

| Submission Format | Main report: .ODT (OpenDocument), Open Sans 12 (or centre standard) Appendices: diagrams, calculations, simulation screenshots/plots, maintenance logs Referencing: Harvard style |
| --- | --- |