BEng (Hons) Electrical & Electronic Engineering (Advanced Standing)

________________________________________________________

Level HE5

_________________________________________________________

Module Title: Introductory Embedded Systems

Module Number: EEE5014

_________________________________________________________

Assignment: 1 of 2

Assignment Title: Group Demonstration of Project Outcomes

_________________________________________________________

Module Tutor:

‘

| Assessment Number | 001 |
| --- | --- |
| Assessment Type (and weighting) | 50% + 50% of the module |
| Submission Date |  |

Aim:

Design, build, test, demonstrate, and professionally evaluate a laboratory pump condition-monitoring and protective-interlock system. This HE5 project builds on introductory work through requirements traceability, integrated sensing and control, structured software, systematic testing, manufacture/sustainability awareness, and communication with technical and non-technical audiences.

Learning outcome

LO1: Apply an integrated or systems approach to the solution of embedded systems problems. (C6)

LO3: Communicate effectively on embedded systems matters with technical and non-technical audiences. (C17)

LO4: Adopt an inclusive approach to engineering practice and recognise the responsibilities, benefits and importance of supporting equality, diversity and inclusion. (C11)

Assessment weighting

This assignment represents 50 % of the total assessment for the module.

Use of Generative Artificial Intelligence (GAI) Applications in this Assessment

| AI Status | Application | Notes |
| --- | --- | --- |
| Category A | No GAI tool is permitted. | While grammar and/or spell checkers may be used to correct individual words and sentences, the use of GAI is not allowed. This is because the learning outcomes require you to produce original assessment work without any GAI assistance Any GAI generated content which is presented as your own original work and is not acknowledged will be assessed for academic misconduct. |

Declaration: At the end of the assessment, you should also include a declaration of any software tools including Generative AI (GAI) applications that you used in developing and completing the assessment.

# PART 1: Group Demonstration of Project

An engineering teaching laboratory uses a recirculating low-voltage process-water rig to demonstrate flow and pressure. A technician currently relies on visual checks to identify low flow, an empty reservoir, overheating, or loose sensor connections. The School requires a condition-monitoring prototype that presents clear status, logs events, and inhibits a simulated pump system when a credible abnormal condition occurs.

Your team is an embedded-systems consultancy asked to deliver a prototype and explain how it could mature into a maintainable product. The system must distinguish normal, warning, and fault states; support controlled recovery; and communicate clearly to operators and technicians.

The prototype is a low-voltage educational demonstrator. It may use a small DC pump, motor simulator, LEDs, or tutor-approved substitute. It must not control a mains-powered motor or be represented as a safety-certified industrial protection system.

Minimum technical requirements:

- Use an Arduino Uno R3, ESP32-S1, ESP32-C3, or tutor-approved microcontroller platform with a documented development toolchain.

- Acquire at least three inputs, including two sensors or one sensor plus a realistic, simulated analogue/digital condition.

- Use at least one digital and one analogue or timed/PWM interface.

- Drive an operator-facing status interface and a low-voltage pump/motor simulator or inhibit output.

- Implement start-up/self-check, normal, warning, fault/inhibited, and recovery states.

- Record timestamped or sequence-numbered events by serial output, non-volatile storage, or approved equivalent.

- Demonstrate a communication interface beyond program upload: UART logging, I2C, SPI, Bluetooth, Wi-Fi, or CAN simulation where facilities permit.

- Demonstrate unit testing, system testing, fault injection or safe simulation, and requirement-to-test traceability.

- Evaluate manufacturability, maintainability, RoHS/WEEE, power use, and end-of-life disposal.

## Platform Guidance

Select one controller and justify its fitness for the completed system. Arduino Uno R3, ESP32-S1, and ESP32-C3 are common laboratory options. Compare the required I/O, ADC channels, timers/PWM, memory, communication interfaces, power constraints, development environment, and cost against your requirements.

An ESP32 platform may be appropriate where wireless communication, greater processing capability, or expanded peripherals are justified, but its 3.3 V logic limits and credential/data-handling implications must be designed for. An Arduino Uno R3 may be appropriate for a simpler wired design, but limitations around memory, connectivity, and I/O expansion must be considered. Document voltage compatibility and any level shifting, driver stage, or pull-up requirements.

## Team Working Expectations

Teams normally contain three to five students. Teams appoint a coordinator and agree a working structure while retaining shared responsibility for technical quality. Keep evidence of planning, review, version control, peer support, inclusive participation, and decisions. Tutors may question any group member to confirm their technical understanding and contribution.

# Task 1.1: Requirements, Traceability, and Architecture

Prepare and present a design package containing:

- Stakeholders, operating context, system boundary, assumptions, constraints, and a clear separation between monitoring, control, and safety-inhibit functions.

- At least eight functional and six non-functional requirements, each with an identifier, measurable acceptance criterion, priority, verification method, and assigned system state where relevant.

- Requirements-traceability matrix linking each requirement to design elements and at least one planned test. Include requirements for start-up/self-check, normal monitoring, warning, fault inhibition, recovery, event logging, and operator information.

- Measurable trigger definitions for each warning/fault. State the sensor condition, simulated input, delay/debounce where used, expected status indication, event-log entry, and output/inhibit response.

- Block diagram showing sensors, interfaces, controller, storage/communications, HMI, actuator/inhibit output, and power subsystem.

- Hardware architecture identifying voltage domains, signal types, buses, protective components, common ground, and the method used to isolate or drive the low-voltage motor/pump simulator.

- Software architecture showing modules, state machine, timing approach, data flow, validity checking, fault latching, event logging, and recovery/reset logic.

- Reasoned selection of controller, sensors, communications, and storage/display components using official sources and stated trade-offs. Include a component/interface table with voltage, protocol, accuracy/range or resolution where relevant, and design rationale.

# Task 1.2: Build and Demonstrate the Prototype

During the live demonstration, show a planned sequence rather than isolated features. Each team member must explain or demonstrate a meaningful part of the system. Show:

- Start-up and self-check behaviour, including the expected safe output state before valid readings are accepted.

- Acquisition, scaling/validation, and presentation of each required input; show serial output, display, or log evidence for at least one live reading.

- Normal state with meaningful status information and the enabled output state.

- Warning condition and correct response, including the exact trigger condition and any changed HMI indication.

- Safely simulated fault that latches the fault/inhibited state, changes the pump/motor simulator output to safe condition, records an event, and communicates the condition.

- Controlled recovery that shows why accidental restart cannot occur. For example, show a valid-condition check followed by a deliberate reset or acknowledge action.

- The additional communication interface or logged data, including an example event record with time/sequence, state, and cause.

- Relevant code structure for sensing, state/decision logic, outputs, communication/logging, and fault handling. Be ready to locate the responsible function/module and explain its inputs and outputs.

Bring code, diagrams, circuit/wiring record, and test evidence. Tutors may ask any member to explain a design choice, demonstrate a subsystem, or explain fault diagnosis.

# Task 1.3: Test, Evaluate, and Communicate Responsibly

Show systematic evidence of:

- Unit tests for significant code modules, interfaces, or subsystems. Include a test identifier, purpose, inputs/setup, expected output, observed output, and evidence reference.

- System test plan covering every functional requirement, normal/boundary operation, communication/logging, warning/fault states, sensor disconnection/implausible values where safe, restart, and recovery.

- Expected and observed results, pass/fail decisions, evidence references, corrective actions, and a requirement-traceability link.

- At least two integration issues, their root causes, resolutions, retest outcomes, and the design/process lesson learned.

- Evaluation of reliability, response time, data validity, maintainability, cost, board/platform limitations, and limitations of the prototype compared with a deployable industrial product.

- Short non-technical explanation for a technician covering benefits, operating limitations, status indicators, and actions for warning/fault states.

- Accessible HMI decisions, such as clear labels, colour-independent status information, understandable alarms, accessible positioning, and plain-language instructions.

- How equitable participation was enabled in planning, task allocation, review, practical work, and the live demonstration.

- Sustainability evaluation covering component choice, power use in normal/fault states, repairability, avoiding unnecessary electronics, RoHS awareness, WEEE disposal, and reuse/recycling.

- Clear acknowledgement that this prototype does not replace formal industrial safety systems, risk assessment, or compliance validation.

Deliverables:

- Live group demonstration: all team members must attend and participate. Tutor-approved recorded contingency evidence may be used only where a live demonstration is impossible.

- Demonstration pack: one PDF containing the slides or display-board content submitted through Moodle/Turnitin as directed by the tutor. Normally this should contain no more than 12 content slides.

- Technical project archive: versioned source code, circuit/wiring record, pin map, and relevant original design files in the format directed by the tutor.

- Operator guide: a concise, non-technical guide explaining normal operation, warning/fault indications, and required operator action.

Use filenames such as EEE5014_A1_TeamName_DemoPack.pdf and EEE5014_A1_TeamName_Code.zip.

Submission Guidelines:

Report Structure

You must demonstrate competent analytical skills which should be evidenced by the appropriate use of calculations and reasoned arguments. The reports should be presented carefully, with accurate spelling, punctuation and word processed and referenced using the Harvard System.

Organise the demonstration pack in the following order:

- Cover slide/page with module, assessment, project title, team member names/numbers, tutor, and demonstration date.

- Task 1.1: Requirements, traceability, and hardware/software architecture.

- Task 1.2: Prototype build, integration, and live operating evidence.

- Task 1.3: Testing, evaluation, communication, inclusive practice, and sustainability.

- Team contribution summary.

- Reference list and source/code acknowledgements.

- Appendices or supplementary files: detailed circuit/wiring record, pin map, code, test records, and operator guide.

Minimum Secondary Research Source Requirements:

Use at least eight relevant, credible sources. Include official datasheets/documentation for the selected platform and key components, plus appropriate sources on testing, sustainability, RoHS/WEEE, accessibility, and manufacture.

The submission deadline is given below

|  | 21/03 | 04/04 | 18/04 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Task 1.1 |  |  |  |  |  |  |
| Task 1.2 |  |  |  |  |  |  |
| Task 1.3 |  |  |  |  |  |  |

### Marking Scheme

| Section | Marks |
| --- | --- |
| Requirements, traceability, and integrated architecture | 20 |
| Prototype functionality, integration quality, and demonstration | 25 |
| Testing, fault handling, debugging, and technical evaluation | 20 |
| Communication with technical and non-technical audiences | 15 |
| Inclusive design, teamwork, and sustainable practice | 10 |
| Professional organisation, evidence, and responses to questions | 10 |
| Total | 100 |

References - Brindley, K. (2005) Practical Electronics Construction: Techniques, Equipment and Projects. Newnes.

- Frenzel, L.E. (2020) Practical Electronic Design for Experimenters. McGraw-Hill Education.

- Horowitz, P. and Hill, W. (2015) The Art of Electronics. 3rd edn. Cambridge University Press.

- Monk, S. (2023) Programming Arduino: Getting Started with Sketches. 3rd edn. McGraw-Hill.

- Official controller, sensor, interface, storage, and actuator datasheets/technical documentation.

- Brindley, K. (2005) Practical Electronics Construction: Techniques, Equipment and Projects. Newnes.

- Frenzel, L.E. (2020) Practical Electronic Design for Experimenters. McGraw-Hill Education.

- Horowitz, P. and Hill, W. (2015) The Art of Electronics. 3rd edn. Cambridge University Press.

Guidelines for the Preparation and Submission of Written Assessments

- Written assessments should be word-processed in Arial or Calibri Light font size 12. There should be double-spacing, and each page should be numbered.

- This practical demonstration has no word count. Limit the demonstration pack to 12 content slides and use the 15–25-minute presentation period effectively. Speaker notes, appendices, source code, figures, tables, and references are not included in the slide limit. Edit or remove this guidance before publishing the brief.

- There should be a title page identifying the programme name, module title, assessment title, your student number, your marking tutor and the date of submission.

- You should include a word-count at the end of the assessment (excluding references, figures, tables and appendices).

Where a word limit is specified, the following penalty system applies:

- Up to 10% over the specified word length = no penalty

- 10 – 20% over the specified indicative word length = 5 marks subtracted (but if the assessment would normally gain a pass mark, then the final mark to be no lower than the pass mark for the assessment).

- More than 20% over the indicative word length = if the assessment would normally gain a pass mark or more, then the final mark will be capped at the pass mark for the assessment.

- All written work should be referenced using the standard University of Bolton referencing style– see: https://www.bolton.ac.uk/library/Study-Skills/Referencing/Home.aspx

- Unless otherwise notified by your Module Tutor, electronic copies of assignments should be saved as word documents and uploaded into Turnitin via the Moodle class area. If you experience problems in uploading your work, then you must send an electronic copy of your assessment to your Module Tutor via email BEFORE the due date/time.

- Please note that when you submit your work to Moodle, it will automatically be checked for matches against other electronic information. The individual percentage text matches may be used as evidence in an academic misconduct investigation (see Section 13).

- Late work will be subject to the penalties:

  - Up to 7 calendar days late = 10 marks subtracted but if the assignment would normally gain a pass mark, then the final mark to be no lower than the pass mark for the assignment.

  - More than 7 calendar days late = This will be counted as non-submission and no marks will be recorded.

Where assessments are graded Pass/Fail only, they will not be accepted beyond the deadline date for submission and will be recorded as a Fail.  Students may request an extension to the original published deadline date as described below.

- In the case of exceptional and unforeseen circumstances, an extension of up to 14 days after the assessment deadline may be granted. This must be agreed by your Programme Leader, following a discussion with the Module Tutor.  You should complete an Extension Request Form available from your Tutor and attach documentary evidence of your circumstances, prior to the published submission deadline.

Extensions over 14 calendar days should be requested using the Mitigating Circumstances procedure, with the exception of extensions for individual projects and artefacts which, at the discretion of the Programme Leader, may be longer than 14 days.

Requests for extensions which take a submission date past the end of the module (normally in week 15) must be made using the Mitigating Circumstances procedures.

Some students with registered disabilities will be eligible for revised submission deadlines. Revised submission deadlines do not require the completion extension request paperwork.

Please note that the failure of data storage systems is not considered to be a valid reason for an extension. It is therefore important that you keep multiple copies of your work on different storage devices before submitting it.

Specific Assessment Criteria:

First class (70% and above):

Students will apply advanced problem-solving skills, technical knowledge and understanding, to establish rigorous and creative solutions that are fit for purpose for all aspects of the problem including production, operation, maintenance, and disposal. Their work will comprehensively address all aspects of DC and AC machine performance, including simulation accuracy, motor operation, efficiency, and system optimization. They will critically evaluate the commercial, economic, and social implications of electrical machine applications and effectively apply engineering knowledge to real-world scenarios involving electrical drives and power systems. Research demonstrating the use of an extensive range of current secondary research sources will be evident. Academic style and references will be excellent.

Second class (50-69%):

Students will apply advanced problem-solving skills, technical knowledge and understanding, to establish rigorous and creative solutions that are fit for purpose for all aspects of the problem including production, operation, maintenance, and disposal. Their analysis will cover key aspects of motor simulation, performance evaluation, and the practical applications of DC and AC machines. They will assess the commercial and economic impacts of electrical machines and demonstrate a broad understanding of relevant engineering principles. Research demonstrating the use of a wide range of current secondary research sources will be evident. Academic style and references will be good.

Third class (40-49%):

Students will apply advanced problem-solving skills, technical knowledge and understanding, to establish rigorous and creative solutions that are fit for purpose for all aspects of the problem including production, operation, maintenance, and disposal. Their analysis will focus on essential aspects of motor performance and evaluation, demonstrating a general understanding of engineering applications in power systems. They will provide an overview of the commercial and economic implications of electrical machines. Research demonstrating the use of a range of current secondary research sources will be evident. Academic style and reference will be fair.

Fail (39% and below): Students who do not meet the requirements of a third-class grade will not successfully complete the assessment activity.

APPENDIX