Lab Workbook EEE4012

Akbar; 10/19/26; EEE4012.

Contents

1.	Measuring a Two-Source DC Resistor Network	2

Aim	2

What you will learn	2

Components and equipment	2

Important polarity note	3

Circuit connections	3

Before you start	3

Step-by-step procedure	4

Predicted results	5

Results	5

Simulation Evidences:	6

Hardware Evidences:	7

E-series values and substitutions	8

Troubleshooting	8

Safety and good practice	8

Review questions	9

# Measuring a Two-Source DC Resistor Network

### Aim

Build the circuit shown and measure its node voltages. Compare the readings with values calculated using Kirchhoff’s Current Law (KCL).

### What you will learn

- How to build a circuit from a schematic.

- How to identify the ground reference and measure node voltages.

- How to check the polarity of voltage sources.

- Why the resistor fitted in the hardware may differ from the value on the schematic.

- How to compare measured results with calculated predictions.

### Components and equipment

- One 6 V DC source

- One 12 V DC source

- One 33 kΩ resistor

- Two 47 kΩ resistors

- One 62 kΩ resistor

- One 20 kΩ resistor

- Digital multimeter

- Breadboard and connecting wires

The resistor values are nominal values. Use the actual measured values in your calculations if your instructor asks for a more accurate comparison.

### Important polarity note

In the attached diagram, the longer plate of the 6 V source appears at the top, so the top terminal is positive. The longer plate of the 12 V source appears on the node 2 side, so node 2 is positive relative to node 1.

### Circuit connections

Use the bottom wire in the schematic as the circuit’s ground reference. This is the point against which the node voltages are measured.

Make these connections:

- Connect the negative terminal of the 6 V source to ground.

- Connect the positive terminal of the 6 V source to one end of the 33 kΩ resistor.

- Connect the other end of the 33 kΩ resistor to node 1.

- Connect one 47 kΩ resistor from node 1 to ground.

- Connect the 12 V source between node 1 and node 2. Its negative terminal goes to node 1; its positive terminal goes to node 2.

- Connect the 62 kΩ resistor from node 2 to ground.

- Connect the other 47 kΩ resistor from node 2 to one end of the 20 kΩ resistor.

- Connect the other end of the 20 kΩ resistor to ground.

The right-hand 47 kΩ and 20 kΩ resistors are in series. Their shared connection is the junction measured in the results table.

Important: The 12 V source connects between nodes 1 and 2. Neither of its terminals should be connected directly to ground.

### Before you start

- Make sure both sources are switched off or disconnected.

- Find the ground rail on your breadboard. If your board has separate ground rails, connect them together so the whole bottom conductor is one common reference.

- Check that each resistor is in the correct breadboard rows. Resistor leads that share a connected row are electrically joined; make sure the resistor is not accidentally bypassed.

- If you are unsure whether a breadboard row is connected internally, check its layout or ask your instructor.

- Do not use a resistance measurement on a powered circuit.

### Step-by-step procedure

Part A: Check the resistors

- Set the multimeter to resistance mode, usually marked Ω.

- Touch one probe to each resistor lead. For best accuracy, the resistor should be disconnected from the circuit while measuring.

- Record the measured value. A reading close to the marked value is normal; for example, a resistor labelled 62 kΩ may not measure exactly 62.0 kΩ.

- Switch the meter back to voltage mode before connecting the power sources.

Part B: Build the circuit

- With the power off, place the resistors on the breadboard according to the connection list above.

- Use a wire to connect the ground points together along the bottom of the circuit.

- Connect the 6 V source with its positive terminal toward the 33 kΩ resistor and its negative terminal to ground.

- Connect the 12 V source with its negative terminal to node 1 and positive terminal to node 2.

- Before turning anything on, trace each connection against the list. Check especially that node 2 is not accidentally connected to ground.

If using bench power supplies instead of batteries, confirm with your instructor that the 12 V output is isolated or suitable for a floating connection between nodes 1 and 2. Some bench supply outputs share a common or earth reference, which can create an unintended connection.

Part C: Power and measure

- Set the multimeter to DC voltage mode, marked V⎓ or DCV. Choose a range above 12 V if the meter does not select its range automatically.

- Connect the black meter lead to COM and the red lead to the voltage socket, usually marked V.

- Switch on the sources.

- Measure node 1: keep the black probe on ground and touch the red probe to node 1. Record the reading.

- Measure node 2 in the same way.

- Measure the junction between the right-hand 47 kΩ and 20 kΩ resistors, again with the black probe on ground.

- Check the 12 V source directly: put the red probe on node 2 and the black probe on node 1. The meter should read approximately +12 V.

- Switch off the sources before changing any connections.

A negative reading at node 1 is expected for the polarity shown. It does not automatically mean the circuit is wired incorrectly.

### Predicted results

Using the nominal resistor values and the source polarities shown:

- The voltage source gives V2-V1=12 V.

- Applying KCL to the circuit gives:

V1-633 kΩ+V147 kΩ+V262 kΩ+V247 kΩ+20 kΩ=0

The predicted voltages are approximately:

| Measurement point | Predicted voltage to ground |
| --- | --- |
| Node 1 | -2.31 V |
| Node 2 | +9.69 V |
| Junction between the right-hand 47 kΩ and 20 kΩ resistors | +2.89 V |

The 12 V source reading should be approximately +12 V when measured with the red probe on node 2 and the black probe on node 1.

### Results

| Measurement | Predicted | Simulated | Measured |
| --- | --- | --- | --- |
| 6 V source positive terminal to ground | +6.00 V |  |  |
| Node 1 to ground | -2.31 V |  |  |
| Node 2 to ground | +9.69 V |  |  |
| Right-hand resistor junction to ground | +2.89 V |  |  |
| Node 2 to node 1 | +12.00 V |  |  |

### Simulation Evidences:

### Hardware Evidences:

### E-series values and substitutions

Resistors are made in standard preferred-value series called E-series. E12 provides 12 nominal values per decade, while E24 provides 24 and E96 provides 96. The extra choices give smaller gaps between available values.

62 kΩ is a standard E24 value, so it is not inherently unusual. It may simply be missing from a particular lab kit. If the kit only has E12 values, nearby alternatives are 56 kΩ and 68 kΩ. Each differs from 62 kΩ by about 10%, which can noticeably shift the predicted readings.

If you substitute a resistor:

- Write down the value actually fitted.

- Measure its resistance if possible.

- Recalculate the predictions using that value before comparing them with your measurements.

- Do not expect the 62 kΩ predictions above to match exactly when a different resistor is fitted.

For example, using the nominal values and the same source polarity, a 56 kΩ substitute gives approximately V1=-2.51 V and V2=+9.49 V. A 68 kΩ substitute gives approximately V1=-2.14 V and V2=+9.86 V.

### Troubleshooting

- Node 1 reads positive instead of negative: check the 12 V source polarity and confirm its positive terminal goes to node 2.

- The voltage between nodes 1 and 2 is not about 12 V: check that the 12 V source is connected across those two nodes and has not been connected to ground.

- Readings are near zero: check that the sources are switched on and that the ground rail is connected to the circuit.

- Readings are very different from predictions: switch off the sources, then check resistor placement, breadboard rows, source settings, and resistor values.

- The meter shows a minus sign: the red probe may be at a lower voltage than the black probe. This can be a valid reading; check which points the probes are touching.

### Safety and good practice

- Switch off the sources before moving wires.

- Use voltage mode for voltage measurements and connect the meter in parallel between the two points being compared.

- Never use the meter’s current setting directly across a voltage source.

- Never measure resistance while the circuit is powered.

- Keep the 12 V source floating between nodes 1 and 2; do not connect either terminal directly to ground.

### Review questions

- Why is node 1 expected to have a negative voltage relative to ground?

- Why must the 12 V source be connected between nodes 1 and 2 rather than between node 2 and ground?

- What does the longer line in a battery symbol indicate?

- Why might a lab kit use 56 kΩ or 68 kΩ instead of 62 kΩ?

- If a resistor is substituted, what should be changed in the predicted results?